Advantech GeniDAQ WinCE Runtime (Alpha Version)

Date: 1/24/2000

Installing Advantech GeniDAQ for WinCE

System Requirements for Host PC

   OS: Microsoft Windows 98/95, Windows NT 4.0 (SP4 above)

   RAM: 64 MB memroy (minimum)

   Disk Space: 20 MB (minimum)

   CPU: Pentium 200 MHz or higher

   Display: VGA resolution or higher

   Microsoft-compatible mouse
System Requirements for HMI-640S

   OS: Microsoft Windows CE Version 2.12

Installation for Host PC

1. Insert the Advantech GeniDAQ CD into Host PC's CD-ROM drive

2. Run the Advantech GeniDAQ 4.0 installation program at

D:\GeniDAQ\Disk1\Setup.exe

Where D is the drive letter of your CD-ROM drive

3. Copy all files under D:\GeniDAQ.410 directory to the GeniDAQ 4.0 installed path, the default path is C:\Program Files\Advantech\GeniDAQ to overwrite the original files.

Installation for HMI-640

1. Power on HMI-640

2. Download the GeniDAQ Windows CE Runtime into HMI-640

Method 1: Download the program through floppy disk

Copy all files under the D:\GeniDAQCE.Runtime directory on the Host PC to floppy disks, then copy them into \Harddisk\GeniDAQCE directory on HMI-640

Method 2: Download the program through Network server

Copy all files under D:\GeniDAQCE.Runtime directory on the Host PC to the network server, then copy them into \Harddisk\GeniDAQCE directory on HMI-640 by following the steps below:

a. Click Start->Programs->Command Prompt menu

b. Issue the command
[\] Net View Server_name
It generates Logon to Network Server Dialog box, then inputs the user name, password and domain for login the server

c. It shows the directory on the server. Then issue the command
[\] Copy \\Server_Name\…\download files
to \Harddisk\GeniDAQCE directory on HMI-640S. After copying all download files, close Command Prompt.

Running Advantech GeniDAQ for WinCE

1. Build your application with Advantech GeniDAQ 4.10 Builder on Host PC

2. Download your application (*.GNI) to HMI-640 by following the download procedure as above

3. Launch \Harddisk\GeniDAQCE\AdsDeviceInstall.exe under My Computer icon to configure and install devices. The Alpha version supports Demo Board only. 

4. After installing a device, you have to save the configuration to System Registry for next run by clicking

Start->Run menu and type in "regsave" to save it.

5. Launch \Harddisk\GeniDAQCE\GeniDAQCE.exe program under My Computer icon to start the Advantech GeniDAQ Runtime for WinCE.

GeniDAQ for WinCE Run-Time Function Specification

· Support strategy file build on GeniDAQ for Win32 platform 

· Support Run-Time kernel on WinCE

· Support Multi-Threading

· Support TCP/IP networking

· Support OPC Client for WinCE

· Support ADSAPI32 for WinCE driver modal

· Limit support TASK Block

· Limit support DISPLAY Block

· Support security control

· Support Remote Download function

· Do not support Basic Script function

· Do not support DDE function

· Do not support OLE Automation interface

· Do not support C-API interface

System Architecture :


[image: image1.wmf]GeniDAQ for WinCE Run-Time

Kernal

Data Center

I/O Driver

Sub. System

I/O Devices

Strategy File

(*.GNI)

Network

Card

ADSAPI32 for

WinCE Driver

OPC Client for

WinCE

OPC Server for

WinCE

Operation Interface

Sub. System

Security File

(SECURITY.P

W)

I/O Devices

Event Log File

(GENIE.ELF,

GENIE.ELH)

Network

Sub. System

Network

Server

Network

Client


Limit support TASK Block:

Block
Descrition
Win32
WinCE

[AI]
Analog Input
Yes
Yes

[AO]
Analog Output
Yes
Yes

[DI]
Digital Input
Yes
Yes

[DO]
Digital Output
Yes
Yes

[TEMP]
Temperature Measurement
Yes
Yes

[COUNT]
Hardware Counter
Yes
Yes

[ALM]
Hardware Alarm
Yes
Yes

[RS232]
RS-232 Port
Yes
Yes

[PID]
PID Control
Yes
Yes

[ON/OFF]
On/Off Control
Yes
Yes

[SOC]
Single Operator Calculation
Yes
Yes

[AVG]
Average
Yes
Yes

[EV]
Event Counter
Yes
Yes

[BS]
Basic Script
Yes
No

[TM]
Timer(need modify)
Yes
Yes

[TMS]
Time Stamp
Yes
Yes

[LOG]
Log File
Yes
No

[INP]
Input File
Yes
No

[DDEC]
DDE Client
Yes
No

[DDES]
DDE Server
Yes
No

[NETIN]
Network Input
Yes
Yes

[SALM]
Software Alarm
Yes
Yes

[BEEP]
Beep
Yes
Yes

[WAVE]
Conditional Wave File
Yes
No

[SYS]
System Control
No
Yes

Limit support DISPLAY Block:

Block
Description
Win32
WinCE

[BBTN]
Binary Button
Yes
Yes

[CBTN]
Conditional Button
Yes
Yes

[MBTN] 
Menu Button(need modify)
Yes
Yes

[NUMC]
Numerical Control
Yes
Yes

[KNOB]
Knob Control
Yes
Yes

[SLD]
Slider Control
Yes
Yes

[IND]
Indicator Display
Yes
Yes

[NUM]
Numerical Display
Yes
Yes

[CTXT]
Conditional Display
Yes
Yes

[BAR]
Bar Graph Display
Yes
Yes

[MET]
Anameter Display
Yes
Yes

[TRN]
Trend Graph Display
Yes
Yes

[HIST]
Historical Trend Display
Yes
No

[XY]
XY Graph Display
Yes
Yes

[CBMP]
Conditional Bitmap Display
Yes
Yes

[RECT]
Rectangle Drawing Display
Yes
Yes

[ROUND] 
Rounded Rectangle Drawing Display
Yes
Yes

[OVAL]
Oval Drawing Display
Yes
Yes

[POLY]
Polygon Drawing Display
Yes
Yes

[LINE]
Line Drawing Display
Yes
Yes

[TXT]
Text String Display
Yes
Yes

[GROUP]
Group Box Display
Yes
Yes

[EVLOG]
Event Log Display
No
Yes

_1002707809.vsd
ADSAPI32 for WinCE Driver�

OPC Client for WinCE�

OPC Server for WinCE�

GeniDAQ for WinCE Run-Time Kernal�

Data Center�

Operation Interface
Sub. System�

Security File
(SECURITY.PW)�

I/O Driver
Sub. System�

I/O Devices�

I/O Devices�

Event Log File (GENIE.ELF, GENIE.ELH)�


Strategy File
(*.GNI)�

Network
Sub. System�

Network
Card�

Network Server�

Network Client�


